Annex II of Tender Documentation

Technical specifications

Tender for delivery of Ribbon cutting and bending machine

Procurement No. S-05

Offer date: _____________

Requirements of Technical Specifications represent minimum technical requirements for the offered supplies and are not to be changed by Bidder.

Bidder shall complete column „Technical Specifications offered“, defining in detail technical specifications of offered supplies (note: Bidder fills the exact specifications of offered supplies, while avoiding filling the columns only with words “compliant” and, “equivalent„ or “yes”).

If manufacturer/brand/model of supply or machine is stated in items of Payment Schedule and/or Technical specifications the equally valuable supplies can be offered. If the term “equivalent” is not expressly stated when naming the brand, it is considered that equally valuable supplies can be offered.

The Bidder offers equally valuable supplies by stating the name of the equivalent supplies below the existing item of Payment Schedule/Technical specifications.

For offer to be considered as compliant, offered supplies must meet all the requirements presented in the Technical Specifications

| Item no. | Technical Specifications Required | Technical Specifications offered |
| --- | --- | --- |
|  | Set consists of L shape and |  |
| 1 | Horizontal L shape forming machine |  |
| 1.1. | Machine for shaping of copper-based PV ribbons |  |
| 1.2. | Power supply: 220 V |  |
| 1.3. | Forming angles: 90° +/- 5° adjustable |  |
| 1.4. | Forming radius: at least 6-10 mm (wider range is better) |  |
| 1.5. | Usable ribbon thickness: 0,2-0,5 mm |  |
| 1.6. | Processing speed: > 3,5 m/min |  |
| 1.7. | Machine must have feeder for ribbon spools of 8 kg |  |
| 2 | Vertical L and U shape forming machine |  |
| 2.1. | Machine for shaping of copper based PV ribbons |  |
| 2.2. | Power supply: 220 V |  |
| 2.3. | Forming angles: 90° +/- 5° adjustable |  |
| 2.4. | Forming radius: at least 6-10 mm (wider range is better) |  |
| 2.5. | Usable ribbon thickness: 0,2-0,5 mm |  |
| 2.6. | Processing speed: L shape: minimum 10 pcs/min U shape: minimum 5 pcs/min |  |
| 2.7. | Machine must have feeder for ribbon spools of 5 kg |  |
| 3. | General requirements |  |
| 3.1. | Machine must have CE |  |
| 3.2. | User HMI must be in English or Croatian language |  |
| 3.3. | Number of sets: 1 |  |

ON BEHALF OF THE BIDDER:

____________________________________________